GET208 LECTURE NOTE

Module 1

Consideration of equilibrium; composite members, stress-strain relation. Generalized Hooke's law. Stresses and strains due to loading and temperature changes.

CONSIDERATION OF EQUILIBRIUM

Introduction

In the context of strength of materials, equilibrium plays a fundamental role in understanding the behavior of structures under load. The concept of equilibrium is essential for analyzing forces and moments that act on a body to ensure that it remains stable and does not undergo any unanticipated motion or deformation.

Main Content

A breakdown of how equilibrium is considered are presented as follows:

a. Equilibrium Conditions

For an object or a structure to be in equilibrium, both force equilibrium and moment equilibrium must be satisfied.

- Force equilibrium: The sum of all external forces acting on the structure must equal zero. This means there should be no net force causing linear acceleration.

Fx=0, Fy=0, Fz=0

where Fx, Fy, Fz​ are the forces in the x, y and z directions, respectively.

- Moment equilibrium: The sum of all external moments (or torques) acting on the structure must also equal zero to prevent rotational acceleration.

Mx=0, My=0, Mz=0

where Mx, My, Mz​ are the moments about the x, y and z axes, respectively.

b. Applications in Strength of Materials

- Internal Forces: The internal forces (such as axial forces, shear forces, and bending moments) within a structure must also be considered to ensure equilibrium. For example, in a beam subjected to loads, the internal shear force and bending moment must balance the external loads and reactions.

- Stress and Strain: The equilibrium equations help in determining the distribution of stress and strain within the material. Once equilibrium is established, you can proceed to analyze how these internal forces lead to deformations and material failure under various loading conditions.

- Statics of Structural Members: In structural analysis, equilibrium is used to find the reaction forces at supports and the internal forces in structural members like beams, trusses, and frames. These reactions and internal forces are then used to calculate stresses and strains, ensuring the structure can withstand the applied loads without failure.

c. Strength Criteria

To ensure a structure’s strength, the material should be able to resist the internal forces and moments while maintaining equilibrium. Strength criteria such as:

- Maxwell’s Criterion

- Von Mises Stress

- Tresca's Criterion

...are applied to ensure that the material's stress does not exceed the material's yield or ultimate strength, maintaining equilibrium without failure.

d. Types of Equilibrium

- Static Equilibrium: When a structure or body is at rest, or its velocity is constant (no acceleration), it is said to be in static equilibrium.

- Dynamic Equilibrium: When a structure or material is subject to dynamic loading, it may experience varying forces over time, but it is still in equilibrium as long as the sum of forces and moments remains zero at any instant.

Conclusion

In the study of the strength of materials, equilibrium is the cornerstone for analyzing forces, stresses, and deformations. By ensuring both force and moment equilibrium, one can design structures that are safe, stable, and capable of withstanding the forces they will encounter in service without failure.

COMPOSITE MEMBERS

Introduction

In the context of strength of materials, composite members refer to structural elements made up of two or more materials with different mechanical properties that work together to resist loads. These materials are combined in such a way that their individual strengths complement each other, resulting in a member with enhanced overall performance compared to a single material. The analysis of composite members requires understanding how different materials behave under load and how they interact with one another.

Main Content

Types of Composite Members

Composite members can be formed in various ways, and the materials used can have different mechanical properties (such as strength, stiffness, thermal expansion, etc.). Common types of composite members include:

- Concrete-Steel Composite Members: Often used in reinforced concrete beams or columns where steel reinforcement bars (rebar) are embedded in concrete. Steel handles tensile stresses while concrete resists compressive stresses.

- Metallic Composite Members: These are made from two or more metals or metal alloys combined to optimize the material properties, like strength, corrosion resistance, or lightweight characteristics (e.g., aluminum-steel composites).

- Fiber-Reinforced Polymer (FRP) Composites: These are advanced materials often used in aerospace or civil engineering, where fibers (like carbon or glass) are embedded in a polymer matrix to create a lightweight yet high-strength material.

- Wood-Steel Composite Members: In some construction applications, wood can be used alongside steel elements to form composite beams or columns.

Analysis of Composite Members

When analyzing composite members, several principles and approaches must be considered:

a. Load Distribution

The load applied to a composite member is distributed between the materials based on their mechanical properties. The materials with higher stiffness (modulus of elasticity) will carry a greater share of the load. If the materials are arranged in parallel, the force distribution depends on their stiffness (Young’s Modulus) and cross-sectional area.

b. Compatibility of Deformation

For composite members to work effectively, the deformations of the different materials must be compatible. This means that the strains in the materials must be related, considering their moduli of elasticity. For instance:

- If a steel bar and a concrete column are connected in a composite member, both materials will experience different amounts of strain under load, but the strains must be compatible at the interface.

c. Stress and Strain Analysis

The stress distribution in a composite member depends on the interaction of the materials and their respective properties. Each material will have its own stress-strain curve, and the total deformation of the member will depend on the combined behavior of the materials under load. This can be represented using the equation for strain compatibility:

σ1E1=σ2E2=…

where σ1, σ2,… are the stresses in different materials, and E1, E2,… are the moduli of elasticity of the materials.

d. Neutral Axis (in Bending)

In the case of composite beams subjected to bending (such as concrete-steel beams), the neutral axis (the axis along which there is no strain) may shift due to the difference in stiffness of the materials. The neutral axis location is determined by the relative stiffnesses and areas of the materials. The bending stress distribution will be different in the two materials depending on their position relative to the neutral axis.

Important Considerations in Composite Members

- Adhesion or Bonding: The interface between the materials in a composite member is critical. Inadequate bonding or adhesion between materials can lead to delamination, separation, or failure at the interface.

- Thermal Expansion: Different materials in a composite member often have different coefficients of thermal expansion. This can lead to internal stresses when the member is subjected to temperature changes, which is important for the design of materials in varying environmental conditions.

- Failure Modes: The failure of composite members can be influenced by the failure of one or more of the materials. Possible failure modes include:

- Tensile failure of the weaker material.

- Compressive failure of materials under compression.

- Shear failure at the interface.

- Delamination or debonding due to poor bonding or incompatible thermal expansions.

Applications of Composite Members

Composite members are widely used in various fields, including:

- Construction: Reinforced concrete (concrete and steel), pre-stressed concrete, and composite steel-concrete beams or slabs.

- Aerospace: Composite materials like carbon fiber-reinforced polymers are used for aircraft and spacecraft components due to their high strength-to-weight ratio.

- Automotive: The use of fiber composites in car manufacturing for lightweight and strong components.

- Marine and Civil Engineering: Composite materials are used for repair and reinforcement of structural components exposed to harsh environments, like bridges and offshore structures.

Exercise(s)

(1) A steel rod of 3cm diameter is enclosed centrally in a hollow copper tube   of external diameter 5cm and internal diameter of 4cm. The composite bar is then subjected to axial force of 45kN. If the length of each bar is equal to 15cm. Determine:

i. the stresses in the steel rod and copper tube.

ii. load carried by each bar.

Take Esteel = 2.1x105N/mm2 & Ecopper = 1.1x105N/mm2

(Ans: i. σsteel=41.8N/mm2  &  σcopper=21.9N/mm2

ii. Psteel = 29.5kN, Pcopper = 15.5kN)

(2) A compound tube consists of a steel tube 140mm internal diameter and 160mm external diameter and an outer brass tube 160mm internal diameter and 180mm external diameter. The two tubes are of the same length of 140mm. The compound carries an axial force of 900N. Find the stresses and the load carried by each tube and the amount it shortens. Take Esteel = 2x105N/mm2 & Ebrass = 1x105N/mm2.

(Ans: σsteel=121.9N/mm2  σbrass=60.95N/mm2, Psteel = 29.5kN,

Pcopper = 15.5kN and δL=0.085mm)

STRESS-STRAIN RELATIONS AND STRESSES AND STRAINS DUE TO LOADING

Introduction

The strength of a material depends on its ability to sustain a load without undue deformation or failure. One of the most important tests to perform in order to determine this property is the tension or compression test. Although several important mechanical properties of a material can be determined from this test, it is used primarily to determine the relationship between the average normal stress and average normal strain in many engineering materials such as metals, ceramics, polymers, and composites.

Main Content

Stress-Strain Relations

The load and corresponding deformation data are used to calculate various values of the stress and corresponding strain in the specimen. A plot of the results produces a curve called the stress–strain diagram. A typical stress–strain diagram is shown in Figure 1 below.

Figure 1: Conventional Stress-Strain Diagram

Elastic Behavior: Elastic behavior of the material occurs when the strains in the specimen are within the elastic region as shown in Figure 1. The curve is actually a straight line throughout most of the region, so that the stress is proportional to the strain. The material in this region is said to be linear elastic. The upper stress limit to this linear relationship is called the proportional limit, σpl . If the stress slightly exceeds the proportional limit, the curve tends to bend and flatten out as shown. This continues until the stress reaches the elastic limit. Upon reaching this point, if the load is removed the specimen will still return back to its original shape.

Yielding: A slight increase in stress above the elastic limit will cause the material to deform permanently. This behavior is called yielding. The stress that causes yielding is called the yield stress, σY , and the deformation that occurs is called plastic deformation.

Strain Hardening: When yielding has ended, an increase in load can be supported by the specimen, resulting in a curve that rises continuously but becomes flatter until it reaches a maximum stress referred to as the ultimate stress, σu . The rise in the curve in this manner is called strain hardening.

Necking: Up to the ultimate stress, as the specimen elongates, its cross-sectional area will decrease. This decrease is fairly uniform over the specimen’s entire length; however, just after, at the ultimate stress, the cross-sectional area will begin to decrease in a localized region of the specimen. As a result, a “neck” tends to form in this region as the specimen elongates further. Here the stress–strain diagram tends to curve downward until the specimen breaks at the fracture stress, σf.

Strain Energy: As a material is deformed by an external loading, it tends to store energy internally throughout its volume. Since this energy is related to the strains in the material, it is referred to as strain energy. Strain energy is denoted as U. For applications, it is often convenient to specify the strain energy per unit volume (V) of material. This is called the strain-energy density, and it can be expressed as:

u=∆U∆V= 12σϵ

If the material behavior is linear elastic, then Hooke’s law applies, σ=Eϵ, and therefore the above equation can be expressed as:

u= 12σ2E

Modulus of Resilience: When the stress reaches the proportional limit, the strain-energy density is referred to as the modulus of resilience, i.e.

ur= 12σplϵpl=  12σpl2E

Generalized Hooke's Law

The stress-strain diagrams for most engineering materials exhibit a linear relationship between stress and strain within elastic limit. Consequently, an increase in stress causes a proportionate increase in strain. This fact is known as Hooke’s law. It is expressed as σ=Eϵ.

E represents the constant of proportionality, which is called the modulus of elasticity or Young’s modulus. The equation above represents the equation of the initial straight-lined portion of the stress-strain diagram up to the proportional limit. Furthermore, the modulus of elasticity represents the slope of this line. Also, the modulus of elasticity is a mechanical property that indicates the stiffness of a material. Materials that are very stiff, such as steel, have large values of E (Est= 200GPa), whereas soft materials such as vulcanized rubber may have low values (Er= 0.7MPa).

A similar modulus called the shear modulus or the modulus of rigidity relates the shearing stress τ and the shearing strain γ as follows:

G=τγ

Poisson’s ratio: A material loaded in one direction will undergo strains perpendicular to the direction of the load in addition to those parallel to the load. The ratio of the lateral (perpendicular) strain to the longitudinal (axial) strain is called Poisson’s ratio and is given as:

υ=-ϵlateralϵlongitudinal

Consider a bar having an original radius r and length L and subjected to the tensile force P. This force elongates the bar by an amount δ and its radius contracts by an amount δ'. Strains in the longitudinal or axial direction and in the lateral or radial direction are given as follows:

ϵlongitudinal= δL   and ϵlateral= δ'r

Like the modulus of elasticity and the shear modulus G, Poisson’s ratio is also a property of a material. Hence, Poisson’s ratio (υ) is related to E and G by the formula:

E = 2(1 + υ)G

Exercise(s)

- An aluminum specimen has a diameter of 25mm and a length of 250mm. If a force of 165kN elongates the length by 1.2mm, determine the modulus of elasticity. Also determine by how much the force causes the diameter of the specimen to contract.  Take  Gal= 26GPa and σY= 440 MPa.

(Ans: E= 70GPa  & δ'= 0.0416mm)

- A square steel rod 15mmx15mm in section is to carry an axial load (compressive) of 150 kN. Calculate the shortening in a length of 75mm. E= 2.03×108 kN/m2.          (Ans: δ = 0.246mm)

- The following observations were made during a tensile test on a mild steel specimen 50mm in diameter and 180mm long. Elongation with 50kN load (within limit of proportionality) is δ=0.256mm, yield load = 150kN, Maximum load =230 kN, length of specimen at fracture = 232mm. Determine (i) Modulus of Elasticity (ii) Yield point stress (iii) Ultimate Stress (iv) Percentage elongation.

(Ans: (i) E= 17.96GPa (ii) σY= 76.4MPa, (iii) σU = 117.14MPa,

(iv)  δpercentage=28.9%)

(4) A hollow cast-iron cylinder 5m long, 400mm outer diameter and metal thickness of 50mm is subjected to a central load on the top when standing straight. The stress produced is 80,000 kN/m2. Assume Young’s modulus for cast iron is 1.5×108 kN/m2. Determine (i) Magnitude of the load (ii) longitudinal strain produced (iii) Total decrease in length.

(Ans: (i) P= 4398.2kN ii ϵlong = 5.33×10-4, (iii) δ=2.67mm)

STRESSES AND STRAINS DUE TO TEMPERATURE CHANGES

Introduction

When a material is subjected to a temperature change, it tends to expand (heat) or contract (cooling). If free to expand or contract, no stress develops; only strain occurs. However, if restrained, thermal stresses are induced.

Main Content

Thermal Stress and Strain

If the temperature of a body is lowered or raised, its dimensions will decrease or increase correspondingly. If these changes are checked, the stresses thus developed in the body are called temperature stresses and the corresponding strains are called temperature strains.

If  L = length of a bar of uniform cross-section

t1 = Initial temperature of the bar

t2 = Final temperature of the bar

α = Coefficient of linear expansion

The extension (e) in the bar due to rise in temperature will be:

e = α(t2 - t1)L

If this elongation in the bar is prevented by some external force or by fixing the bar ends, the temperature strain thus produced will be given by:

Temperature strain = α(t2 - t1)LL = α(t2 - t1)

Therefore, temperature stress developed = α(t2 - t1)E

Important Points:

- If both ends of a bar are fixed and temperature rises, compressive stress develops.

- If temperature decreases, tensile stress develops.

- In partially restrained members, both stress and deformation occur.

- Different materials in composite structures may experience differential expansion, causing internal stresses

Applications in Engineering

- Bridges (expansion joints)

- Railway tracks

- Pipelines

- Structures exposed to temperature variation

Exercise(s)

(1) A steel rod 15m long is at a temperature of 15oC. Find the free extension of the length when the temperature is raised to 60oC. Determine the temperature stress produced when:

i. The expansion of the rod is prevented.

ii. The rod is permitted to expand by 6mm. Take α = 12×10-6per oC and

E = 200 GPa

(2) Calculate the values of stress and strain in a non-prismatic steel bar ABC which is rigidly restrained at its ends A and C. The length of both AB and BC is 0.3m and the cross-sectional areas of AB and BC are 400mm2 and 800mm2, respectively. A close fit exists at both of the rigid supports at room temperature and the temperature is raised by 75 oC. Take E = 200 GPa and α = 12×10-6per oC for steel.

Conclusion

Thermal stresses and strains arise when materials undergo temperature changes but are restrained from expanding or contracting freely. While thermal strain occurs naturally with temperature variation, thermal stress is induced only under restriction. Understanding these concepts is critical in structural and mechanical design to prevent failure, cracking, or distortion in components exposed to temperature fluctuations. Proper consideration of material properties, thermal expansion, and suitable allowances like expansion joints ensures durability and safety in engineering structures.

References/Further Readings

- Hibbeler R.C. (2011) “Mechanics of Materials”, Prentice Hall, 8th edition.

- Riley W.F. and Zachary L. (1989) “Introduction to Mechanics of Materials”, John Wiley & Sons, 1st edition.

- Riley W.F., Sturges L. and Morris D. (2007) “Mechanics of Materials”, John Wiley & Sons, 6th edition.

- Dias da Silva V. (2006) “Mechanics and Strength of Materials”, Springer, 1st edition.

- Hearn E.J. (1997) “Mechanics of Materials I”, Butterworth-Heinemann, 3rd edition.

- Vasilier V.V. and Morozov E.V. (2001) “Mechanics and Analysis of Composite Materials”, Elsevier Science Ltd, 1st edition.